# Network Configuration Documentation: VLANs, OSPF, DHCP, and Remote Access

Packet Tracer lab, 10.3.16.0/21 address block

## 1. Overview

The network has two switches, three routers, one DHCP server and eight PCs. The 10.3.16.0/21 block is divided into four VLANs and three point-to-point links. Each switch connects to its router with a single cable, so inter-VLAN routing is done with router-on-a-stick (a trunk on the switch and subinterfaces on the router). OSPF (area 0) connects the three routers. The DHCP server sits behind Router1, so Router0 and Router2 relay DHCP requests with ip helper-address.

Topology (left to right):

PC0, PC1 (VLAN 10) and PC2, PC3 (VLAN 11) connect to Switch0 (SW-A / SW-1), which connects to Router0. Router0 connects to Router1, which also connects to Server0 (DHCP) above it. Router1 connects to Router2, which connects to Switch1 (SW-B / SW-2). PC4, PC5 (VLAN 20) and PC6, PC7 (VLAN 21) connect to Switch1.

In the final diagram, all link lights are green.

## 2. Addressing plan

| Segment | Network | Mask | Router address |
| --- | --- | --- | --- |
| VLAN 10 (BLUE) | 10.3.16.0/22 | 255.255.252.0 | 10.3.16.1 (Router0 gi0/0.10) |
| VLAN 11 (RED) | 10.3.20.0/23 | 255.255.254.0 | 10.3.20.1 (Router0 gi0/0.11) |
| Router0 to Router1 | 10.3.22.192/30 | 255.255.255.252 | R0 .193, R1 .194 |
| Router1 to Router2 | 10.3.22.196/30 | 255.255.255.252 | R1 .197, R2 .198 |
| Router1 to Server0 | 10.3.22.200/30 | 255.255.255.252 | R1 .201, Server .202 |
| VLAN 21 (ORANGE) | 10.3.22.0/25 | 255.255.255.128 | 10.3.22.1 (Router2 gi0/1.21) |
| VLAN 20 (GREEN) | 10.3.22.128/26 | 255.255.255.192 | 10.3.22.129 (Router2 gi0/1.20) |

Note: the diagram label for the whole block reads 255.255.248.0, which is correct for a /21.

### PC addressing

The PCs were given static addresses. The table shows the plan (the DHCP pools would hand out the same range).

| PC | VLAN | IP address | Mask | Gateway |
| --- | --- | --- | --- | --- |
| PC0 | 10 | 10.3.16.2 | 255.255.252.0 | 10.3.16.1 |
| PC1 | 10 | 10.3.16.3 | 255.255.252.0 | 10.3.16.1 |
| PC2 | 11 | 10.3.20.2 | 255.255.254.0 | 10.3.20.1 |
| PC3 | 11 | 10.3.20.3 | 255.255.254.0 | 10.3.20.1 |
| PC4 | 20 | 10.3.22.130 | 255.255.255.192 | 10.3.22.129 |
| PC5 | 20 | 10.3.22.131 | 255.255.255.192 | 10.3.22.129 |
| PC6 | 21 | 10.3.22.2 | 255.255.255.128 | 10.3.22.1 |
| PC7 | 21 | 10.3.22.3 | 255.255.255.128 | 10.3.22.1 |

## 3. Assumptions

- PCs are on fa0/1 to fa0/4 of each switch, and the uplink to the router is gi0/1.

- Router0 uses gi0/0 toward SW-A and gi0/1 toward Router1. Router1 uses gi0/0 toward Router0, gi0/1 toward Router2, and gi0/2 toward the server. Router2 uses gi0/0 toward Router1 and gi0/1 toward SW-B.

- The assignment's R-A2 is Router2.

- "10 ports per VLAN" was read as 10 ports for each VLAN on its switch.

- The server is 10.3.22.202, and Router1's server-facing port is 10.3.22.201.

## 4. Build order

The work was done from the bottom layer up so each step could be tested: PCs, then switches, then routers, then the server, then the assignment's extra requirements (security, port allocation, access control, MOTD).

## 5. Step 1: PCs

On each PC: Desktop, IP Configuration, Static, then the address, mask and gateway from the table in section 2.

## 6. Step 2: Switch0 (SW-A)

VLANs, access ports and trunk. Only the commands after the prompt were typed.

enable

configure terminal

no ip domain-lookup

hostname SW-A

vlan 10

name VLAN10

exit

vlan 11

name VLAN11

exit

interface range fa0/1 - 2

switchport mode access

switchport access vlan 10

exit

interface range fa0/3 - 4

switchport mode access

switchport access vlan 11

exit

interface gi0/1

switchport mode trunk

end

write memory

Verified with show vlan brief (VLAN 10 on Fa0/1 and Fa0/2, VLAN 11 on Fa0/3 and Fa0/4) and show interfaces trunk.

Troubleshooting note: the first attempt failed with "Invalid input" because the whole prompt (for example SW-1(config-vlan)#exit) was typed as part of the command. Only the text after the # should be entered.

## 7. Step 3: Switch1 (SW-B)

enable

configure terminal

no ip domain-lookup

hostname SW-B

vlan 20

name VLAN20

exit

vlan 21

name VLAN21

exit

interface range fa0/1 - 2

switchport mode access

switchport access vlan 20

exit

interface range fa0/3 - 4

switchport mode access

switchport access vlan 21

exit

interface gi0/1

switchport mode trunk

end

write memory

## 8. Step 4: Router0

Subinterfaces for VLAN 10 and 11, the link to Router1, and OSPF.

enable

configure terminal

no ip domain-lookup

hostname Router0

interface gi0/0

no shutdown

exit

interface gi0/0.10

encapsulation dot1Q 10

ip address 10.3.16.1 255.255.252.0

ip helper-address 10.3.22.202

exit

interface gi0/0.11

encapsulation dot1Q 11

ip address 10.3.20.1 255.255.254.0

ip helper-address 10.3.22.202

exit

interface gi0/1

ip address 10.3.22.193 255.255.255.252

no shutdown

exit

router ospf 1

network 10.3.16.0 0.0.3.255 area 0

network 10.3.20.0 0.0.1.255 area 0

network 10.3.22.192 0.0.0.3 area 0

end

write memory

The SW-A to Router0 link light turned green once gi0/0 was brought up with no shutdown. Router interfaces are shut down by default, so a link stays red until both ends are up.

## 9. Step 5: Router1

enable

configure terminal

no ip domain-lookup

hostname Router1

interface gi0/0

ip address 10.3.22.194 255.255.255.252

no shutdown

exit

interface gi0/1

ip address 10.3.22.197 255.255.255.252

no shutdown

exit

interface gi0/2

ip address 10.3.22.201 255.255.255.252

no shutdown

exit

router ospf 1

network 10.3.22.192 0.0.0.3 area 0

network 10.3.22.196 0.0.0.3 area 0

network 10.3.22.200 0.0.0.3 area 0

end

write memory

## 10. Step 6: Router2 (initial configuration)

enable

configure terminal

no ip domain-lookup

hostname Router2

interface gi0/0

ip address 10.3.22.198 255.255.255.252

no shutdown

exit

interface gi0/1

no shutdown

exit

interface gi0/1.20

encapsulation dot1Q 20

ip address 10.3.22.129 255.255.255.192

ip helper-address 10.3.22.202

exit

interface gi0/1.21

encapsulation dot1Q 21

ip address 10.3.22.1 255.255.255.128

ip helper-address 10.3.22.202

exit

router ospf 1

network 10.3.22.196 0.0.0.3 area 0

network 10.3.22.0 0.0.0.127 area 0

network 10.3.22.128 0.0.0.63 area 0

end

write memory

## 11. Step 7: Server0 (DHCP)

Desktop, IP Configuration: IP 10.3.22.202, mask 255.255.255.252, gateway 10.3.22.201.

Services, DHCP: service on, one pool per VLAN.

| Pool | Default gateway | Start IP | Mask | Max users |
| --- | --- | --- | --- | --- |
| VLAN10 | 10.3.16.1 | 10.3.16.2 | 255.255.252.0 | 1000 |
| VLAN11 | 10.3.20.1 | 10.3.20.2 | 255.255.254.0 | 500 |
| VLAN20 | 10.3.22.129 | 10.3.22.130 | 255.255.255.192 | 60 |
| VLAN21 | 10.3.22.1 | 10.3.22.2 | 255.255.255.128 | 120 |

Each pool starts just after its gateway so a PC never receives the router's address. Max users is kept below the usable host count of each subnet.

## 12. Step 8: Assignment requirements

### 12.1 Switch changes (VLAN names by color, 10 ports per VLAN, unused ports shut down, remote access)

VLANs were renamed to match the diagram colors: VLAN 10 BLUE, VLAN 11 RED, VLAN 20 GREEN, VLAN 21 ORANGE. Each VLAN received 10 ports (its two original PC ports plus eight more). Leftover ports were shut down. A management IP on a VLAN interface allows Telnet to the switch.

SW-A:

enable

configure terminal

vlan 10

name BLUE

exit

vlan 11

name RED

exit

interface range fa0/1 - 2 , fa0/5 - 12

switchport mode access

switchport access vlan 10

exit

interface range fa0/3 - 4 , fa0/13 - 20

switchport mode access

switchport access vlan 11

exit

interface range fa0/21 - 24 , gi0/2

shutdown

exit

interface vlan 10

ip address 10.3.16.250 255.255.252.0

no shutdown

exit

ip default-gateway 10.3.16.1

enable secret testi2026

line console 0

password testi2026

login

exit

line vty 0 15

password testi2026

login

end

write memory

SW-B:

enable

configure terminal

vlan 20

name GREEN

exit

vlan 21

name ORANGE

exit

interface range fa0/1 - 2 , fa0/5 - 12

switchport mode access

switchport access vlan 20

exit

interface range fa0/3 - 4 , fa0/13 - 20

switchport mode access

switchport access vlan 21

exit

interface range fa0/21 - 24 , gi0/2

shutdown

exit

interface vlan 21

ip address 10.3.22.126 255.255.255.128

no shutdown

exit

ip default-gateway 10.3.22.1

enable secret testi2026

line console 0

password testi2026

login

exit

line vty 0 15

password testi2026

login

end

write memory

### 12.2 Passwords

Every connection type (console, vty, and aux on routers) and the enable mode use the password testi2026. Enable mode uses enable secret, so the password is stored encrypted.

### 12.3 Router0: passwords and VLAN 11 restriction

Requirement: VLANs 10, 20 and 21 reach each other, while VLAN 11 may only communicate with VLAN 10. OSPF already routes all networks, so two extended access lists on the VLAN 11 subinterface restrict it.

enable

configure terminal

enable secret testi2026

line console 0

password testi2026

login

exit

line aux 0

password testi2026

login

exit

line vty 0 15

password testi2026

login

exit

ip access-list extended VLAN11-IN

permit ip 10.3.20.0 0.0.1.255 10.3.20.0 0.0.1.255

permit ip 10.3.20.0 0.0.1.255 10.3.16.0 0.0.3.255

permit udp any eq 68 any eq 67

deny ip any any

exit

ip access-list extended TO-VLAN11

permit ip 10.3.16.0 0.0.3.255 10.3.20.0 0.0.1.255

deny ip any any

exit

interface gi0/0.11

ip access-group VLAN11-IN in

ip access-group TO-VLAN11 out

end

write memory

- VLAN11-IN filters traffic leaving VLAN 11. It allows VLAN 11 to itself, VLAN 11 to VLAN 10, and DHCP requests.

- TO-VLAN11 filters traffic entering VLAN 11. Only VLAN 10 is allowed in.

### 12.4 Router1: passwords

enable

configure terminal

enable secret testi2026

line console 0

password testi2026

login

exit

line aux 0

password testi2026

login

exit

line vty 0 15

password testi2026

login

end

write memory

### 12.5 Router2 (R-A2): SSH, passwords and MOTD

Secure access is required only on R-A2, so its vty lines accept SSH only and log in with a local user.

enable

configure terminal

hostname R-A2

ip domain-name test.local

enable secret testi2026

username admin secret testi2026

crypto key generate rsa modulus 1024

ip ssh version 2

line console 0

password testi2026

login

exit

line aux 0

password testi2026

login

exit

line vty 0 15

login local

transport input ssh

exit

banner motd #Test was a good practice of VLANs, OSPF, ACLs and SSH. Tricky but fun.#

end

write memory

SSH login: user admin, password testi2026. The MOTD banner contains short feedback about the test.

## 13. Testing and verification

| Test | Command or action | Expected result |
| --- | --- | --- |
| VLANs and ports | show vlan brief on both switches | 10 ports per VLAN, correct names |
| Trunks | show interfaces trunk | Uplink trunking |
| Interfaces | show ip interface brief on routers | Configured interfaces up/up |
| OSPF | show ip ospf neighbor on Router1 | Two neighbors |
| Routes | show ip route on Router0 | O routes for VLAN 20 and 21 |
| VLAN 10 to 20 and 21 | Ping PC0 to PC4, PC0 to PC6 | Success |
| VLAN 20 to 21 | Ping PC4 to PC6 | Success |
| VLAN 10 to 11 | Ping PC0 to PC2 | Success |
| VLAN 11 to 20 or 21 | Ping PC2 to PC4 | Fails (blocked by ACL) |
| VLAN 20 or 21 to 11 | Ping PC4 to PC2 | Fails (blocked by ACL) |
| Telnet to a switch | telnet 10.3.16.250 from PC0 | Password prompt |
| SSH to R-A2 | ssh -l admin 10.3.22.198 | Login, MOTD shown |
| Telnet to R-A2 | telnet 10.3.22.198 | Refused |
| ACL counters | show access-lists on Router0 (in privileged mode) | Match counters increase |

## 14. Problems encountered and fixes

- Invalid input on the switch: the prompt was typed along with the command. Fix: type only what follows the #.

- Empty show interfaces trunk: the trunk was set on the wrong or not yet connected port. Fix: check show interfaces status, set the trunk on the connected uplink, and bring up the router side.

- Red link light between switch and router: router interfaces are shut down by default. Fix: no shutdown on the router interface.

- show access-lists rejected: the router was in user mode (Router0>) after the console password was added. Fix: type enable and the enable password first.

## 15. Final state

All links are green, VLANs are named and populated, OSPF connects the three routers, DHCP relay is in place, every line and enable mode is protected with testi2026, SSH is enforced on R-A2, VLAN 11 is limited to VLAN 10, unused ports are shut down, and R-A2 shows a MOTD banner.

Done By

Nelson Mbua Mosisah

Cybersecurity Analyst, Network Engineer.